AQI

This repository provides tools and scripts to track, analyze, and visualize air quality data. It includes features for fetching real-time data from APIs, calculating AQI, and generating reports or dashboards. Supports multiple data sources and provides customizable thresholds for different regions and pollutants.

🚨 Air Quality Monitoring System with Arduino and ThingSpeak

This project uses multiple gas sensors with an Arduino UNO and an Ethernet Shield to measure air quality and upload the data to ThingSpeak for real-time visualization and analysis.

📋 Table of Contents

- Description

- Hardware Components

- Software and Libraries

- Wiring Diagram

- Installation and Setup

- Code Explanation

- ThingSpeak Dashboard

- Usage

📑Description

This project reads data from multiple MQ-series gas sensors and a DHT-11 temperature/humidity sensor. The collected data is sent to ThingSpeak for online monitoring and visualization.

Supported Sensors:

- MQ-9: CO, LPG, Methane

- MQ-8: Hydrogen (H₂)

- MQ-5: LPG, CO, Propane

- MQ-135: Smoke, CO₂

- MQ-4: Methane, CNG

- MQ-6: LPG, Propane

- DHT-11: Temperature and Humidity

🛠️ Hardware Components

- Arduino UNO

- Ethernet Shield (W5100)

- Gas Sensors (MQ-9, MQ-8, MQ-5, MQ-135, MQ-4, MQ-6)

- DHT-11 Temperature/Humidity Sensor

- Jumper Wires

- Breadboard

💻 Software and Libraries

- Arduino IDE

- ThingSpeak Library

- Ethernet Library

Required Libraries:

Install these libraries using the Arduino Library Manager (Sketch -> Include Library -> Manage Libraries):

- ThingSpeak

- Ethernet

🔌 Wiring Diagram

| Sensor | Arduino Pin |
| --- | --- |
| MQ 5 | Analog Pin A4 |
| MQ 135 | Analog Pin A0 |
| MQ 4 | Analog Pin A5 |
| MQ 6 | Analog Pin A3 |
| MQ 8 | Analog Pin A2 |
| MQ 9 | Analog Pin A1 |
| DHT-11 | Digital Pin D2 |
| Ethernet Shield | Follows standard SPI pins on the UNO |

🚀 Installation and Setup

- Clone the Repository:

- git clone https://github.com/yourusername/air-quality-monitor.git

cd air-quality-monitor

- Configure ThingSpeak:

  - Sign up at ThingSpeak.

  - Create a new channel and note the Channel ID and Write API Key.

  - Update the following constants in the code with your details:

  - #define SECRET_CH_ID 2731380                   // Your ThingSpeak Channel ID

#define SECRET_WRITE_APIKEY "YOUR_WRITE_API_KEY"

- Upload Code to Arduino:

  - Connect the Arduino to your PC.

  - Open the project in the Arduino IDE.

  - Select the correct board and port (Tools -> Board -> Arduino UNO).

  - Click Upload.

📝 Code Explanation

- Gas Sensors Configuration:
Reads values from different MQ sensors, calculates gas concentrations in ppm, and sends data to ThingSpeak.

- Ethernet Configuration:
Uses DHCP or a static IP to connect the Ethernet Shield to the network.

- ThingSpeak Integration:
Sends sensor readings to ThingSpeak fields for real-time monitoring.

ThingSpeak Fields Mapping:

| Field | Gas Measurement |
| --- | --- |
| Field 1 | Hydrogen (H₂) |
| Field 2 | LPG |
| Field 3 | Smoke |
| Field 4 | Carbon Monoxide (CO) |
| Field 7 | Propane |

📊 ThingSpeak Dashboard

Log in to ThingSpeak to visualize real-time graphs and analyze trends based on your sensor data.

▶️ Usage

- Power the Arduino and ensure the Ethernet Shield is connected to the network.

- Open the Serial Monitor (9600 baud) to see real-time sensor readings.

- Visit your ThingSpeak channel to view the data visualization.